RS38 FW update SOP

0. download and update qualcomn drive.

https://qualcommusbdrivers.com/downloads/qualcomm-v1-0-10061-1-usb-driver/

1. PC: Unzip the attached file “platform-tools-latest-windows”.

2. RS38: go to "usb debug mode".

3. RS38: connect usb cable to PC.

4. RS38: Enable “Allow USB Debuggind" & “always allow from this computer”.

5.Type “CMD” in the search bar of the unzip folder.

*After entering CMD it should look like this.

6.Type “adb root” then press enter.

7.type “adb reboot edl” then press enter.

now it’s in download mode.

8.Click “Configuration” to do setups.

###Do not enable Erase All Before Download###

9.Click “Select Port” and select 9008.

10.Select “rawprogram0_no_persist.xml, rawprogram2.xml, rawprogram3.xml, rawprogram4.xml, rawprogram5.xml”.